Digitalization of Procurement Processes: Application of Lean Methods to Improve Efficiency

Manuel Inácio, Vitor Anes, and António Abreu

Abstract This study investigates the application of lean methodologies in optimizing the internal material requisition process within a company, with a focus on waste reduction and operational efficiency. The subject of the study is a small to medium-sized enterprise (SME) based in Greater Lisbon, specializing in industrial machinery maintenance and repair, equipment rental, and accessory manufacturing. Several inefficiencies were identified, including redundancies in the current requisition process, resulting in excessive rework, resource consumption, and operational delays. These inefficiencies disrupt workflows, affect material requisition timelines, and create operational bottlenecks, ultimately hindering business opportunities and service delivery. To address these challenges, the following tasks were undertaken: (1) a detailed analysis of the existing procurement process was conducted, followed by the development of a simulation model to evaluate various improvement scenarios; (2) idle periods were identified and analyzed using Value Stream Mapping (VSM) to highlight areas with potential for efficiency gains; (3) a digitalized process simulation was created to compare against the existing process; (4) a feasibility study for implementing the digitalized process was performed. The results indicate that the proposed improvements will significantly enhance process efficiency, leading to positive impacts on the company's overall operations.

Keywords Material requisition · Industrial processes · Lean · Waste reduction · Digitalization · Simulation

# 1 Introduction

In today’s highly competitive and globalized business environment, organizations increasingly focus on strategic areas such as sustainability, digitalization, and Industry 4.0. The implementation of lean methodologies has become widely recognized as a vital strategy for reducing waste and optimizing production processes, aligning with the evolving industrial landscape. First introduced by John Krafcik in 1988 [1], lean production aims to create a streamlined system — defined as "a set of interconnected elements organized coherently to achieve a specific goal" [2]. By applying lean methodologies, organizations can enhance the efficiency of production systems while minimizing waste, ultimately gaining a competitive advantage through higher productivity [3].

The lean philosophy emphasizes a systemic approach, where all production elements are interconnected and harmonized to achieve common objectives. This holistic perspective is essential for understanding and optimizing the overall performance of production systems [4]. Lean tools such as Value Stream Mapping (VSM) are particularly effective for visualizing and analyzing value streams within organizations, helping to identify inefficiencies and eliminate waste [5]. Additionally, scenario-based simulations play a crucial role in process optimization, allowing companies to model virtual environments, identify bottlenecks, and test improvements without disrupting actual production or making premature investments. Software like Arena facilitates these simulations, driving better process efficiency and decision-making [6].

...

2 Background

2.1 Lean methodologies in material requisition

Lean methodologies, particularly techniques such as Value Stream Mapping (VSM) and Kanban, have been widely adopted to streamline processes like material requisition. According to Womack and Jones [10], the primary objective of lean is the continuous identification and elimination of waste across all stages of production. These methodologies help identify inefficiencies in material flow, communication, and procurement processes. Recent studies have indicated that integrating lean techniques such as VSM can significantly optimize material requisition processes by reducing lead times, minimizing inventory levels, and improving overall operational flow [11, 12].

...

4 Improvement Hypotheses

Upon analyzing the company’s current purchasing procedure and simulating it using Arena software (Fig. 1), three improvement hypotheses were proposed.

Fig. 1. Current internal requisition process flow simulated in Arena.

...

5.2 Performance evaluation metrics

The results obtained for all the models are presented in Table 1.

Table 1. Comparison of results.

| Models / Indicators (average values from replications) | Current Model | Hypothesis 1 | Hypothesis 2 | Hypothesis 3 |
| --- | --- | --- | --- | --- |
| Request arrivals | 400 | 454 | 438 | 441 |
| Total requests leaving the process | 329 | 341 | 409 | 426 |
| Orders generated | 272 | 45 | 48 | 48 |
| Time spent by the Purchasing Department (%) | 55 | 9 | 10 | 10 |
| Number of requests authorized by the requestor's supervisor | 396 | 385 | 374 | 384 |
| Time spent by the requestor's supervisor (%) | 41 | 40 | 39 | 40 |
| Number of requests arriving at the warehouse | 387 | 363 | - | - |
| Time spent by the warehouse stock checker (%) | 78 | 73 | - | - |
| Number of requests arriving to the purchasing responsible for authorization | 286 | 259 | 82 | 83 |
| Time spent by the purchasing responsible in authorizations (%) | 99 | 90 | 19 | 20 |
| Total Time in System (in hours) | 4,08 | 9,94 | 4,98 | 3,78 |
| Process Efficiency (%) [Total requests leaving the process / Request arrivals × 100] | 82,25 | 75,11 | 93,38 | 96,60 |

...

Considering Tables 3 and 5, and using Formula 1, the Payback Period (PP) was determined for each scenario of both proposals, yielding the following results:

where, PP is the Payback Period; n is the year immediately before the investment amortization shifts from negative to positive, which, for this analysis, is 0 for all scenarios; CFn is the initial investment (a negative amount, i.e., - €3,000); CFn+a is the amortized value for year n+1 (in this case, year 1), which, for each scenario, is: €9,558 for the pessimistic, €12,472 for the probable, and €15,385 for the optimistic scenarios. Substituting values into the formula for each scenario, we get:

...

6 Conclusion

This study explored three different ways to improve the internal requisition process of an industrial SME, focusing on digitalization, the implementation of a Production Planning and Control (PPC) system, and the integration of lean methodologies with Industry 4.0 technologies. The results clearly showed that all three approaches helped reduce overall processing time, with the third hypothesis proving to be the most efficient. From a financial perspective, while the second hypothesis had the highest Internal Rate of Return (IRR), the third one offered the best long-term financial stability, as indicated by its superior Net Present Value (NPV) across all scenarios. Although it requires a longer payback period, its long-term benefits outweigh the wait. Given this balance between operational improvements and financial strength, the third hypothesis stands out as the best approach for ensuring sustainable growth and continuous innovation, aligning with both lean principles and Industry 4.0 advancements. Beyond this specific case, the findings of this study can be adapted and applied to other industrial SMEs facing similar challenges in their requisition and procurement processes. Many businesses, regardless of their sector or location, struggle with inefficiencies caused by outdated systems, lack of digital integration, and process variability. By embracing digital tools, automation, and structured methodologies like lean and Industry 4.0, companies can significantly improve efficiency, reduce waste, and enhance decision-making. These improvements are particularly relevant for businesses that rely on well-coordinated procurement and resource management.

...

References

- Holweg, M. (2007). The genealogy of lean production. Journal of Operations Management, 25(2), 420–437. https://doi.org/10.1016/j.jom.2006.04.001

- Meadows, D. H. (2008). Thinking in systems: A primer. Chelsea Green Publishing.

- Souza, R., Ferenhof, H., & Forcellini, F. (2022). Industry 4.0 and Industry 5.0 from the lean perspective. International Journal of Management, Knowledge and Learning, 11, 145–155. https://doi.org/10.53615/2232-5697.11.145-155

- Dennis, P. (2009). Produção lean simplificada [Simplified lean production]. Bookman Editora.

- Romero, L. F., & Arce, A. (2017). Applying value stream mapping in manufacturing: A systematic literature review. IFAC-PapersOnLine, 50(1), 1075–1086. https://doi.org/10.1016/j.ifacol.2017.08.385

- de Paula Ferreira, W., Armellini, F., & De Santa-Eulalia, L. A. (2020). Simulation in Industry 4.0: A state-of-the-art review. Computers & Industrial Engineering, 149, 106868. https://doi.org/10.1016/j.cie.2020.106868

- Brealey, R. A., Myers, S. C., & Allen, F. (2014). Principles of corporate finance (11th ed.). McGraw-Hill Education.

- Hillier, D., Ross, S., Westerfield, R., Jaffe, J., & Jordan, B. (2019). Corporate finance (4th European ed.). McGraw Hill Education.

- Lefley, F. (1996). The payback method of investment appraisal: A review and synthesis. International Journal of Production Economics, 44(3), 207–224. https://doi.org/10.1016/0925-5273(96)00022-9

- Womack, J. P., & Jones, D. T. (1996). Beyond Toyota: How to root out waste and pursue perfection. Harvard Business Review, 74(5), 140–151.

- Liu, Q., & Yang, H. (2020). An improved value stream mapping to prioritize lean optimization scenarios using simulation and multiple-attribute decision-making method. IEEE Access, 8, 204914–204930. https://doi.org/10.1109/ACCESS.2020.3037660

- Jha, N., & Prashar, D. (2022). Adoption of Industry 4.0 in lean manufacturing. In Industrial Internet of Things (pp. 107–127). CRC Press. https://doi.org/10.1201/9781003102267-6

- Tay, H. L., & Loh, H. S. (2022). Digital transformations and supply chain management: A Lean Six Sigma perspective. Journal of Asia Business Studies, 16(2), 340–353. https://doi.org/10.1108/JABS-10-2020-0415

- Valamede, L. S., & Akkari, A. C. S. (2020). Lean 4.0: A new holistic approach for the integration of lean manufacturing tools and digital technologies. International Journal of Mathematical, Engineering and Management Sciences, 5(5), 851. https://doi.org/10.33889/IJMEMS.2020.5.5.066

- Zafarzadeh, M., Wiktorsson, M., & Baalsrud Hauge, J. (2021). A systematic review on technologies for data-driven production logistics: Their role from a holistic and value creation perspective. Logistics, 5(2), 24. https://doi.org/10.3390/logistics5020024

- Bechtsis, D., Tsolakis, N., Iakovou, E., & Vlachos, D. (2022). Data-driven secure, resilient and sustainable supply chains: Gaps, opportunities, and a new generalised data sharing and data monetisation framework. International Journal of Production Research, 60(14), 4397–4417. https://doi.org/10.1080/00207543.2021.1957506

- Cifone, F. D., Hoberg, K., Holweg, M., & Staudacher, A. P. (2021). ‘Lean 4.0’: How can digital technologies support lean practices? International Journal of Production Economics, 241, 108258. https://doi.org/10.1016/j.ijpe.2021.108258

- Wang, C.-N., Vo, T. T. B. C., Hsu, H.-P., Chung, Y.-C., Nguyen, N. T., & Nhieu, N.-L. (2024). Improving processing efficiency through workflow process reengineering, simulation and value stream mapping: A case study of business process reengineering. Business Process Management Journal, 30(7), 2482–2515. https://doi.org/10.1108/BPMJ-11-2023-0869

- van Weele, A. J. (2018). Purchasing and supply chain management (7th ed.). Cengage Learning EMEA.

- Carayannis, E. G., & Popescu, D. (2005). Profiling a methodology for economic growth and convergence: Learning from the EU e-procurement experience for Central and Eastern European countries. Technovation, 25(1), 1–14. https://doi.org/10.1016/S0166-4972(03)00071-3

- Gunasekaran, A., & Ngai, E. W. T. (2004). Information systems in supply chain integration and management. European Journal of Operational Research, 159(2), 269–295. https://doi.org/10.1016/j.ejor.2003.08.016

- Mahi, R. (2024). Optimizing supply chain efficiency in the manufacturing sector through AI-powered analytics. International Journal of Management Information Systems and Data Science, 1(1), 41–50. https://doi.org/10.62304/ijmisds.v1i1.116

- Perboli, G., Musso, S., & Rosano, M. (2018). Blockchain in logistics and supply chain: A lean approach for designing real-world use cases. IEEE Access, 6, 62018–62028. https://doi.org/10.1109/ACCESS.2018.2875782

About Authors

Vitor Anes – PhD, PhD, Prof., Instituto Superior de Engenharia de Lisboa of the Polytechnic University of Lisbon (Rua Conselheiro Emídio Navarro, 1, 1959-007, Lisboa, Portugal)

e-mail: vitor.anes@isel.pt

https://orcid.org/0000-0002-8526-398X

António Abreu – PhD, Prof., Instituto Superior de Engenharia de Lisboa of the Polytechnic University of Lisbon (Rua Conselheiro Emídio Navarro, 1, 1959-007, Lisboa, Portugal)

e-mail: antonio.abreu@isel.pt

https://orcid.org/0000-0001-8839-5606

Manuel Inácio – MSc, Instituto Superior de Engenharia de Lisboa of the Polytechnic University of Lisbon (Rua Conselheiro Emídio Navarro, 1, 1959-007, Lisboa, Portugal)

e-mail: A36949@alunos.isel.pt

https://orcid.org/0000-0001-8839-5606

M. Inácio

Polytechnic University of Lisbon

1 Rua Conselheiro Emídio Navarro, 1959-007 Lisboa, Portugal

e-mail: A36949@alunos.isel.pt

https://orcid.org/0000-0001-8839-5606

V. Anes (✉)

Polytechnic University of Lisbon

1 Rua Conselheiro Emídio Navarro, 1959-007 Lisboa, Portugal

e-mail: vitor.anes@isel.pt

https://orcid.org/0000-0002-8526-398X

A. Abreu

Polytechnic University of Lisbon

1 Rua Conselheiro Emídio Navarro, 1959-007 Lisboa, Portugal

e-mail: antonio.abreu@isel.pt

https://orcid.org/0000-0001-8839-5606